# CAN Device Python SDK – API Reference

This document provides a detailed reference for the CAN Device Python SDK. It describes the initialisation sequence, the hardware management APIs, the configuration controls, and the CAN communication methods exposed by the SDK.

# 1. Initialisation

## __init__()

Initialises the CAN SDK and loads the underlying DLL. This function allocates internal resources and validates communication with the CAN hardware.

Raises:

RuntimeError – If the DLL fails to load or the hardware initialisation fails.

# 2. Hardware Management

## get_channel()

Retrieves the active communication channel associated with the CAN device.

Returns:

str – COM port or channel name, or None if no active channel is available.

## open()

Opens the connection to the CAN device and prepares it for configuration and operation.

Raises:

RuntimeError – If the device fails to open.

## close()

Closes the active connection to the CAN device. Communication is disabled until the device is reopened.

## deinit()

Releases all DLL handles and internal SDK resources. This method should be called before application termination.

# 3. Configuration & Control

## set_bitrate(bitrate)

Configures the CAN bus baud rate. This method must be called before starting the CAN controller.

Parameters:

bitrate (uint32) – CAN baud rate in bits per second

Supported CAN bit rates: 10 kbps, 20 kbps, 50 kbps, 83.3 kbps, 100 kbps, 125 kbps, 250 kbps, 500 kbps, 750 kbps, 1 Mbps

## start()

Starts the CAN controller and enables message transmission and reception.

## stop()

Stops CAN message processing while keeping the device connection open.

# 4. Communication Methods

## transmit(can_id, data, ext=False)

Transmits a CAN frame onto the bus.

Parameters:

can_id (int) – CAN identifier.

data (bytes) – Payload length from 1 to 8 bytes.

ext (bool) – Set to True for extended (29-bit) identifiers; False for standard (11-bit).

Returns:

int – DLL return code (0 indicates successful transmission).

## read()

Performs a non-blocking read of a single CAN frame from the receive buffer.

Returns:

dict – CAN frame data, or None if no message is available.

Return Format:

{
  "id": <int>,
  "dlc": <int>,
  "data_hex": <str>,
  "ext": <bool>,
  "error": <bool>,

"rtr": <bool>
}

## read_timeout(timeout)

Blocks until a CAN frame is received or the specified timeout expires.

Parameters:

timeout (uint8) – Timeout duration in milliseconds.

Returns:

dict – CAN frame data, or None if the timeout expires.

Return Format:

{
  "id": <int>,
  "dlc": <int>,
  "data_hex": <str>,
  "ext": <bool>,
  "error": <bool>,
  "rtr": <bool>
}

## read_withMask(filter_id, mask_id, timeout)

Reads a CAN frame that matches the specified CAN ID filter and mask.

Parameters:

filter_id (int) – Target CAN identifier.

mask_id (int) – Bitmask applied to the filter.

timeout (uint8) – Timeout duration in milliseconds.

Returns:

dict – Matching CAN frame, or None if no matching frame is received.

Return Format:

{
  "id": <int>,
  "dlc": <int>,
  "data_hex": <str>,
  "ext": <bool>,
  "error": <bool>,
  "rtr": <bool>
}